Real-Time System Mid-term Project Assignment

Deadline: 27/11/2025

Group members: 3-4 students per group

Submission: PDF report and GitHub link

1. Project Overview

In this project, students will gain hands-on experience with real-time embedded systems by modifying and extending an existing RTOS-based project. You will work with the project YoloUNO_PlatformIO - RTOS_Project, which is implemented on a microcontroller development board. The mission is to develop a new, complete project with functionalities that differ by at least 30% from the original code base.

2. Project Tasks and Requirements

Basically, students can propose additional features of their own, but these must be of a scope and complexity equivalent to the five suggested tasks bellow. All programming and development work must be carried out on the ESP32 S3 platform using the PlatformIO plug-in.

Your group must implement, document, and demonstrate the following tasks, adapting and extending them with new features and logic as outlined below:

Task 1: Single LED Blink with Temperature Conditions

- Redefine the LED blinking behavior to respond to different temperature conditions (at least 3 different behaviors).

- Use semaphores to manage task synchronization in your implementation.

- Ensure that the condition handling and semaphore logic are clearly explained in your code and report.

Task 2: NeoPixel LED Control Based on Humidity

- Redefine NeoPixel (RGB LED) color patterns to represent different humidity levels (at least 3 levels/colors).

- Utilize semaphore synchronization technique for updating and displaying color changes.

- Clearly show the mapping between humidity value ranges and colors.

Task 3: Temperature and Humidity Monitoring with LCD Display

- Define conditions for creating/releasing semaphores based on sensor readings.

- Integrate an LCD module to display real-time temperature and humidity values.

- At least 3 different display states (e.g., normal, warning, critical) according to measurement conditions.

Task 4: Web Server in Access Point Mode

- Redesign the web server interface for better usability.

- Web server must enable control over two devices (e.g., LED1 and LED2, which can be renamed or replaced as appropriate for your group’s application).

- The interface must include at least two buttons and appropriately labeled control actions.

Task 5: TinyML Deployment & Accuracy Evaluation

- Describe the dataset used for model training, including data collection and labeling steps.

- Implement a TinyML model and run it on your microcontroller.

- Measure and evaluate the recognition accuracy of your model on the hardware, and provide discussion and conclusion regarding performance.

3. Submission Requirements

- Report: PDF file detailing the design, implementation, and results of all five tasks. The report must include:

  - Introduction and Objectives

  - Description of functionalities implemented in all 5 tasks

  - Implementation highlights (e.g., semaphore logic, web server redesign, LCD integration, TinyML workflow)

  - Evaluation/Experimental results (especially for TinyML)

  - Group discussion and conclusions

  - Names and roles of all group members

  - GitHub repository link to your group’s source code

- Code: Well-commented, original code. Your codebase must be hosted on GitHub and referenced in your report.

- Deadline: Submit your PDF report via [Insert Submission Portal/Method] by 27/10/2025.

4. Assessment Rubric

| Criteria | Weight |
| --- | --- |
| Functionality and Correctness | 60% |
| Report quality & clarity | 25% |
| Code quality & comments in source code | 5% |
| Innovation & Improvements | 5% |
| Group organization & Contribution | 5% |

Bonus points will be awarded for novel features, creative user interface, or advanced TinyML applications.

5. Regulations

- Each group must have 3-4 members.

- All group members must contribute equally; tasks and contributions should be stated explicitly in the report.

- Plagiarism or direct copying from other groups or sources is strictly prohibited and will result in disqualification.

6. Support and Resources

- Original project resources: https://github.com/nhanksd85/YoloUNO_PlatformIO/tree/RTOS_Project

- For technical issues contact in the Zalo group of the course

7. Timeline

- Project Q&A: Discuss in Zalo Group

- Project submission: 27/11/2025